Part number	Description

A-5100	Pallet Truck - Powered Scissor Lift 21.25" x 45.25"

Pictures:

Detailed description:	The A-5100 series pallet truck offers sturdy construction with hard chromium-plated piston and overload protection. The lowering valve allows a controlled lowering of the load. The rounded fork ends reduce damage to pallets.  Lead-in and lead-out rollers enable easy handling of pallets.  The steering wheels and fork wheels have sealed bearings, which guarantee ease of movement and prevent early wear.

File name: A-5100.jpg

File name: A-5000 2.jpg

Technical information:

|  | A-5100 |
| --- | --- |
| Lifting height max: | 31.5" |
| Lowered fork height: | 3.625" |
| Fork length: | 45.25" |
| Width over forks: | 21.25" |
| Total length: | 61.25" |
| Fork width: | 6.25" |
| Clear width between forks: | 8.75" |
| Steering wheels: | 7.125" x 2" |
| Load wheels (single): | 3" x 2.375" |
| Maximum height: | 48.5" |
| Turning angle: | 2 x 90 degrees |
| Battery: | 12 V |
| Capacity: up to 21.5" | 2200 lbs |
| from 21.5" | 1650 lbs |
| Truck weight: | 378 lbs |

Safety features:		The hydraulic unit and battery are integrated in the shield thus avoiding snagging. A relief valve protects				 against overloading. The scissor lifts are equipped with a burst pipe valve.