Part number	Description

A-5000	Pallet Truck 27" x 48"

A-5010	Pallet Truck 27" x 36"

A-5020	Pallet Truck 27" x 42"

A-5030	Pallet Truck 27" x 63"

Pictures:

File name: A-5000.jpg

File name: A-5000 2.jpg

Detailed description:	The A-5000 series pallet truck offers sturdy construction with hard chromium-plated piston and overload protection. The lowering valve allows a controlled lowering of the load. The rounded fork ends reduce damage to pallets.  Lead-in and lead-out rollers enable easy handling of pallets.  The steering wheels and fork wheels have sealed bearings, which guarantee ease of movement and prevent early wear.

Technical information:

|  | A-5000 A-5010 A-5020 A-5030 |
| --- | --- |
| Fork length: | 48" (standard) 36" 42" 63" |
| Total lifting height: | 7.625" |
| Lowered fork height: | 2.875" |
| Width over the forks: | 20" or 27" |
| Fork width: | 7" x 2" |
| Steering wheel: | 3" x 3.5" |
| Capacity: | 5500 lbs |
| Truck weight: | 176 lbs |

Advantages:	The strong reinforcement of the lower part of the handle prevents damage of the handle even under full load.  The pressure base plate with its double bearing achieves an even distribution of the load and prevents early wear of linkage lever mounts.  The pressure plate serves also as lateral collision protection.  The steering radius of 120 degrees allows steering and handling even in the smallest spaces.

No maintenance and no servicing schedule is required.  Additional greasing is required after extreme use such as applications in wet rooms, or repeated high pressure cleaning.  All this guarantees long life and low service costs.