Part number	Description

A-6000	Stacker - Powered - 31.5"

A-6010	Stacker - Powered - 47.25"

A-6020	Stacker - Powered - 63"

Pictures:

File name: A-6000.jpg

File name: A-6000 2.jpg

Detailed description:	The A-6000 series stackers lift and lower loads electro-hydraulically via a lowering grip mounted at the tiller handle, which ensures quick working cycles. The stacker allows the operator to work in an ergonomic posture while pallets are deposited or retrieved from the floor or from a pallet store. The A-6000 series stackers offer maximum maneuverability due to the chain-operated link of tiller arm movement. The large rollers ensure smooth steering operation. The lowering speed is defined which guarantees a smooth lowering independent of the load.

Technical information:

|  | A-6000 A-6010 A-6020 |
| --- | --- |
| Lift height max: | 31.5" 47.25" 63" |
| Lowered fork height: | 2.875" |
| Fork length: | 29.25" |
| Width over the forks: | 21.25" |
| Total width: | 26" |
| Total length: | 71" |
| Clear width between forks: | 6.25" |
| Steering wheels:* | 7.875" x 2" |
| Load wheels (single/tandem): | 3" x 3.5" / 3" x 2.5" |
| Load center: | 23.5" |
| Electric motor: | 1.1 kw, 12V |
| Battery: | 12V |
| Capacity: | 2200 lbs |
| Total height: | 50" 62.5" 81.375" |
| Total weight with battery: | 634 lbs 671 lbs 713 lbs |
| Additional equipment: | Battery charger |
| *Right-hand steering wheel with parking brake |  |

Safety features:	A load backrest protects the driver as well as the load from touching the chain. A relief valve prevents damage caused by overload.

Your choice of options:	Parking brake on right steering wheel, or parking brake on both wheels.

Automotive pinch weld adaptor